| 3-Part Specification for Prefabricated Tiled Spas and Swim Spas. | NESPA Enterprises, Inc. 2800 Richter Ave., Unit C Oroville, Ca 95965 Voice: (888) 479-4677 Fax: (530) 534-9915 Website: www.tiledspas.com |
| --- | --- |

Section 13170

Prefabricated Tiled Spas

GENERAL NOTES TO SPECIFIER: THIS SPECIFICATION SECTION HAS BEEN PREPARED TO ASSIST DESIGN PROFESSIONALS IN THE PREPARATION OF PROJECT OR OFFICE MASTER SPECIFICATIONS. IT FOLLOWS GUIDELINES ESTABLISHED BY THE CONSTRUCTION SPECIFICATIONS INSTITUTE, AND THEREFORE MAY BE USED WITH MOST MASTER SPECIFICATION SYSTEMS WITH MINOR EDITING. EDIT CAREFULLY TO SUIT PROJECT REQUIREMENTS. MODIFY AS NECESSARY AND DELETE ITEMS THAT ARE NOT APPLICABLE. VERIFY THAT REFERENCED SECTION NUMBERS AND TITLES ARE CORRECT. (NUMBERS AND TITLES REFERENCED ARE BASED ON MASTERFORMAT, 1995 EDITION). THIS SECTION ASSUMES THE PROJECT MANUAL WILL CONTAIN COMPLETE DIVISION 1 DOCUMENTS INCLUDING SECTIONS 01330SUBMITTAL PROCEDURES, 01620PRODUCT OPTIONS, 01630PRODUCT SUBSTITUTION PROCEDURES, 01660PRODUCT STORAGE AND HANDLING REQUIREMENTS, 01740cLEANING, 01770CLOSEOUT PROCEDURES, AND 01780CLOSEOUT SUBMITTALS. CLOSE COORDINATION WITH DIVISION 1 SECTIONS IS REQUIRED. IF THE PROJECT MANUAL DOES NOT CONTAIN THESE SECTIONS, ADDITIONAL INFORMATION SHOULD BE INCLUDED UNDER THE APPROPRIATE ARTICLES. THIS IS A Closed PROPRIETARY SPECIFICATION. NOTES TO THE SPECIFIER ARE CONTAINED IN BOXES AND SHOULD BE DELETED FROM FINAL COPY. OPTIONAL ITEMS REQUIRING SELECTION BY THE SPECIFIER ARE ENCLOSED WITHIN BRACKETS, E.G. [35] [40] [45]. MAKE APPROPRIATE SELECTIONS AND DELETE OTHERS. ITEMS REQUIRING ADDITIONAL INFORMATION ARE UNDERLINED BLANK SPACES, E.G. ___________ . OPTIONAL PARAGRAPHS REQUIRING SELECTION OF ONE OF THE OPTIONS ARE SEPARATED BY “OR” WITHIN A BOX, E.G. OR OPTIONAL PARAGRAPHS AND FEATURES THAT MAY BE SELECTED OR DELETED AS DESIRED ARE SHOWN IN BOLD FACE TYPE. DELETE BOLD FACE TYPE WHEN INCLUDING THESE PARAGRAPHS OR FEATURES. ELECTRONIC VERSIONS OF THIS SPECIFICATION UTILIZE AUTOMATIC PARAGRAPH NUMBERING.

THERE ARE MANY VARIABLES THAT AFFECT SPA SELECTION AND THE ACCOMPANYING EQUIPMENT, COMPONENTS, AND ACCESSORIES. PLEASE CONTACT NESPA ENTERPRISES PRIOR TO DESIGNING AND SPECIFYING A SPA SYSTEM.

# General

## Summary

### Section Includes: Prefabricated [standard] [custom] designed tile surfaced, reinforced fiberglass based [spa] [swimspa], [complete with mechanical equipment] [excluding mechanical equipment].

### Related Sections:

#### 02315 – Excavation and Fill: Excavation for spa.

#### 03300 – Cast-in-Place Concrete: Spa mounting pad.

#### Division 15 Sections for related Plumbing work.

#### Division 16 Sections for related Electrical work.

CERAMIC TILE MATERIAL CAN BE SPECIFIED BY ALLOWANCE IN THE FOLLOWING PARAGRAPH OR BY SPECIFIC CRITERIA IN PART 2. IF SPECIFYING BY ALLOWANCE, INCLUDE CASH AMOUNTS IN THE DIVISION 1 SECTION  COVERING ALLOWANCES AND ENSURE THAT STATEMENTS USED HERE COMPLIMENT THAT SECTION.  IF TILE IS SPECIFIED UNDER PART 2, DELETE FOLLOWING PARAGRAPH.

### Allowances: Ceramic tile material.

INCLUDE APPROPRIATE LANGUAGE BELOW IF PRODUCTS SPECIFIED IN THIS SECTION ARE TO BE BID AS ALTERNATES. OTHERWISE DELETE  FOLLOWING PARAGRAPH.

### Alternates:

#### Reference Section 01230Alternates.

## Submittals

### Reference Section 01330Submittal Procedures; submit following items:

#### Product Data:

##### Complete manufacturer’s data for spa, equipment, components, and accessories.

#### Shop Drawings: Include plan, section, lip detail, tile section detail, jet location detail, plumbing and equipment drawings.

#### Samples:

DELETE FOLLOWING TWO SUBPARAGRAPHS IF TILE AND GROUT COLORS ARE SPECIFIED IN PART 2.

##### Initial Ceramic Tile Color Selection: Tile manufacturer’s standard tile sample color board showing full range of available colors.

##### Initial Grout Color Selection: Grout manufacturer’s standard color card showing full range of available colors.

##### Final Ceramic Tile Color Verification: Minimum 12 x 12 inch (300 x 300 mm) grouted sample board including tile color(s) and grout from initial selections.

#### Quality Assurance/Control Submittals:

##### Certificates: IAPMO Certificate.

##### Manufacturer’s Installation Instructions.

### Closeout Submittals: Reference Section 01780Closeout Submittals; submit following items:

#### Operating and Maintenance Instructions.

#### Special Warranties.

## Quality Assurance

### Certifications: IAPMO certification label set in tile on interior surface of spa.

## Delivery, Storage, and Handling

### Reference Section 01660Product Storage and Handling Requirements.

### Follow manufacturer’s instructions.

## Warranty

### Special Warranty

#### Structural Soundness: 10 years.

#### Tile Adhesion to Fiberglass: 5 years.

#### Epoxy Grout Adhesion: 3 years.

COORDINATE NUMBER OF YEARS FOLLOWING THREE ITEMS ARE TO BE WARRANTED WITH MANUFACTURER’S STANDARD WARRANTY FOR EQUIPMENT SELECTED IN PART 2.

#### Pumps: ___ years.

#### Filters: ___ years.

#### Heaters: ___ years.

#### Lights: 1  year.

#### Cover: 1 year.

## Maintenance

### Extra Materials: Tile and grout repair kit with instructions.

#### Include minimum 4 sf (0.37 m2) of each color of field tile.

#### Include minimum 3 lf (1 m) of each color of trim tile.

### Maintenance Service: Provide [weekly] [bi-monthly] maintenance service for a period of [1] [3] [ __ ] months. Include chemicals, filter cleaning, cleaning and removal of debris from spa.

# Products

## Manufacturer

### NESPA Enterprises, Inc.	Tel: 	(888) 479-4677

### 2800 Richter Ave., Unit C	Fax:	(530) 534-9915

### Oroville, CA 95965	E-mail:	sales@tiledspas.com

### Website:	www.tiledspas.com

ENTER MODEL NAME BELOW FROM CURRENT NESPA “COMMERCIAL” PRODUCT INFORMATION OR WEBSITE, OR SELECT CUSTOM DESIGN

### Model: [ ______________ ] [Custom design as shown on Drawings].

## Materials

ALL TILE SHOWN ON THE NESPA WEBSITE OR INDIVIDUAL MANUFACTURERS WEBSITES IS NOT ALWAYS READILY AVAILABLE. VERIFY AVAILABILITY WITH NESPA PRIOR TO SPECIFYING SPECIFIC TILE AND COLORS. OTHER TILE IN ADDITION TO THOSE LISTED BELOW CAN ALSO BE USED. PLEASE CONTACT NESPA FOR TO CONFIRM ACCEPTABILITY OF ANY OTHER TILE.

DELETE FOLLOWING PARAGRAPH IF A TILE ALLOWANCE IS SPECIFIED IN PART 1.

### Tile:

#### Manufacturer: [Dal Tile, Keystone Series] [American Olean, Unglazed Ceramic Mosaics Series] [ ______________ ].

#### Size: [1 x 1 inch (25 x 25 mm)] [2 x 2 inch (50 x 50 mm)] [ _________ ]

#### Minimum wet slip coefficient of friction: 0.60 per ASTM C 1028.

TILE COLORS CAN BE SPECIFIED BELOW, SHOWN ON DRAWINGS, OR SELECTED DURING SUBMITTAL REVIEW. IF SHOWN ON DRAWINGS, DELETE BOTH ALTERNATE PARAGRAPHS BELOW.

#### Color:

##### Field: _______________ .

##### Water Line: ________________ .

##### Step and Bench Edges: ______________ .

##### Horizontal Lip: _______________ .

OR

3.	Color: As selected by Architect.

### Grout:

#### Manufacturer: C-Cure

#### Type: ColorSet Epoxy 931

#### Color: [ ____________ ] [As selected by Architect].

### Sand.

### Miscellaneous plumbing and electrical materials as required to complete final connections.

SELECTION OF EQUIPMENT, COMPONENTS, AND ACCESSORIES IN THE FOLLOWING THREE ARTICLES ARE DEPENDENT ON MANY VARIABLES. PLEASE CONTACT NESPA ENTERPRISES PRIOR TO SPECIFYING THESE ITEMS AND VERIFY THAT DRAWINGS SHOW AT LEAST A SCHEMATIC LAYOUT INCLUDING QUANTITIES OF THE VARIOUS ITEMS.

## Equipment

### Pumps:

#### Booster Pump: __________________________.

#### Filter/Heater Pump: ___________________________.

### Filter: ________________________.

### Heater: ____________________________.

## Components

### Skimmer: ____________________________.

### Suction Fittings: ____________________________.

### Jets: ____________________________.

### Valves: ____________________________.

### Controls: ____________________________.

### Lights: [SpaBrite 100 w with stainless steel facing ring] [SpaBrite 100 w with plastic facing ring].

## Accessories

### Handrails: 0.065 inch (1.65 mm) Stainless steel tubing; [catalog no. __________ ] [configuration as shown on Drawings].

### Cover:

#### Sunstar Spa Top.

##### Safety Requirements: Comply with ASTM F 1346.

COLOR CAN BE PRESELECTED FROM THE NESPA WEBSITE OR SELECTED DURING SUBMITTAL REVIEW.

#### Color: [ ____________  ] [As selected by Architect].

# Execution

## Examination

### Examine substrates upon which spa will be installed.

#### Verify that concrete pad is in accordance with spa manufacturer’s requirements.

### Coordinate with responsible entity to correct unsatisfactory conditions.

### Commencement of work by installer is acceptance of substrate conditions.

## INSTALLATION

MANUFACTURER'S  INSTALLATION INSTRUCTIONS ARE ADEQUATE FOR NORMAL INSTALLATION CONDITIONS. UNUSUAL CONDITIONS MAY REQUIRE ADDITIONAL INFORMATION IN THIS ARTICLE.

### Install products in accordance with manufacturer's installation instructions including final plumbing and electrical connections, leak testing, and backfill.

#### Keep spa full of water for remainder of construction period.

### Install cover in accordance with cover manufacturer’s installation instructions.

VERIFY THAT WORK EXCLUDED IN THE FOLLOWING PARAGRAPH IS SPECIFIED IN OTHER SECTIONS.

### Interface with Adjacent Deck or Surface Materials: Work of this section terminates at the outer edge of the spa lip. Sealant joints or other transitions shall be installed under sections.

## Cleaning

### Reference Section 01740Cleaning.

### Cleaning requirements within three days of [Substantial Completion] [Final Acceptance by Owner]:

#### Remove water from spa.

#### Remove accumulated dirt and debris and sponge clean.

#### Refill spa with clean water.

## Demonstration

### Train Owner’s Representative in proper operation and maintenance of spa and equipment.

## Protection

### Protect tile surfaces and equipment from damage during remainder of construction period.

End of Section

This specification was prepared specifically for NESPA Enterprises, Inc. by ASC Specification Consultants. Comments or suggestions for improvement should be addressed to: ASC; 8687 Great Horned Owl Lane; Blaine, WA 98230-6306; Tel: (360) 371-5487. E-mail: specifier@asc-specs.com

Issue Date: May 1, 2003